Experimental study on the extraction of methane hydrate in silica sand by dissolving

Litao Chen1, Baojiang Sun1*, Ningtao Zhang1, Wantian Zhou1, Haotian Wang1, Ye Chen1, Hailong Lu2*

1 Key Laboratory of Unconventional Oil and Gas Development, Ministry of Education, China University of Petroleum (East China), Qingdao 266580, China

2 School of Engineering, Peking University, Beijing 100871, China

Abstract: Natural gas hydrate (NGH) is a type of ice-like compounds formed by natural gas and water at low temperatures and high pressures. As estimated and partially revealed by worldwide geological survey, NGH widely distributes in the sediments of seafloor and permafrost zones. NGH has gained wide attention and become research hotspot due to its potential as an energy reserve. Among the resources, NGH in the shallow layer or directly exposed on the seafloor has been widely discovered, and its formation process and stability law are still unclear. In order to reveal its stability law, the dissolving process of methane hydrate in quartz sand was experimentally studied. The dissolving efficiencies of methane hydrate in silica sand have been studied by injecting water and oil, respectively. The results show that both water and mineral oil can effectively dissolve methane hydrate in quartz sand, with the gas-to-liquid volume ratio about 2 for water injection and about 10 for oil injection. The cumulative gas production of water dissolving generally increases logarithmically with time, and the gas production rate fluctuates greatly, but it decreases gradually as a whole, and the gas-liquid ratio is stable at first and then decreases gradually; the cumulative gas production of oil dissolving generally increases in S shape with time, and the gas-liquid ratio increases first and then decreases gradually. At the same injection rate, the gas production rate and gas-liquid ratio of oil dissolving are significantly higher than those of water dissolution. The dissolving rate of hydrate is mainly affected by the type of dissolving medium (water or oil, in this work) and the contact status between dissolving medium and hydrate. Water and oil are easily channeled into quartz sand, forming a dominant seepage channel. The gas-to-liquid ratio decreases when water/oil forms dominant seepage channel. The results provide theoretical basis for stability studies of NGH in the shallow layer or directly exposed on the seafloor. It seems feasible in principle to exploit gas hydrate in the shallow layer or exposed to the sea floor by dissolving. It is suggested that systematic experimental and numerical simulation studies should be continued in the future.

Keywords: methane; hydrate; dissolving; silica sand; experiment

* Corresponding author.

Prof. Sun Baojiang, E-mail: sunbj1128@vip.126.com

Prof. Lu Hailong, E-mail: hlu@pku.edu.cn